Material Safety Data Sheet

Phosphorous Acid, Crystalline

ACC# 17125

Section 1 - Chemical Product and Company Identification

MSDS Name: Phosphorous Acid, Crystalline 
Catalog Numbers: A246-500, A24JOHN 
Synonyms: Phosphonic acid; Phosphorous acid. 
Company Identification:
              Fisher Scientific
              1 Reagent Lane
              Fair Lawn, NJ 07410 
For information, call: 201-796-7100 
Emergency Number: 201-796-7100 
For CHEMTREC assistance, call: 800-424-9300 
For International CHEMTREC assistance, call: 703-527-3887

| Section 2 - Composition, Information on Ingredients |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | CAS# | Chemical Name | Percent | EINECS/ELINCS |  |
|  | 10294-56-1 | Phosphorous acid | >97 | 233-663-1 |  |

Section 3 - Hazards Identification

EMERGENCY OVERVIEW

Appearance: white or yellow crystals.
Danger! Causes eye and skin burns. May cause severe respiratory tract irritation with possible burns. May cause severe digestive tract irritation with possible burns. Air sensitive. Hygroscopic (absorbs moisture from the air). 
Target Organs: Lungs, eyes, skin, mucous membranes. 

Potential Health Effects 
Eye: Contact with liquid is corrosive to the eyes and causes severe burns. 
Skin: Contact with liquid is corrosive and causes severe burns and ulceration. May cause skin rash (in milder cases), and cold and clammy skin with cyanosis or pale color. 
Ingestion: Causes gastrointestinal tract burns. 
Inhalation: Irritation may lead to chemical pneumonitis and pulmonary edema. Causes severe irritation of upper respiratory tract with coughing, burns, breathing difficulty, and possible coma. Aspiration may lead to pulmonary edema. May cause systemic effects. 
Chronic: Effects may be delayed. Repeated contact may cause corneal damage.

Section 4 - First Aid Measures

Eyes: In case of contact, immediately flush eyes with plenty of water for a t least 15 minutes. Get medical aid immediately. 
Skin: In case of contact, immediately flush skin with plenty of water for at least 15 minutes while removing contaminated clothing and shoes. Get medical aid immediately. Wash clothing before reuse. 
Ingestion: If swallowed, do NOT induce vomiting. Get medical aid immediately. If victim is fully conscious, give a cupful of water. Never give anything by mouth to an unconscious person. 
Inhalation: If inhaled, remove to fresh air. If not breathing, give artificial respiration. If breathing is difficult, give oxygen. Get medical aid. 
Notes to Physician: Treat symptomatically and supportively.

Section 5 - Fire Fighting Measures

General Information: As in any fire, wear a self-contained breathing apparatus in pressure-demand, MSHA/NIOSH (approved or equivalent), and full protective gear. During a fire, irritating and highly toxic gases may be generated by thermal decomposition or combustion. Use water spray to keep fire-exposed containers cool. Substance is noncombustible. Contact with metals may evolve flammable hydrogen gas. 
Extinguishing Media: For small fires, use water spray, dry chemical, carbon dioxide or chemical foam. Do NOT get water inside containers. For large fires, use dry chemical, carbon dioxide, alcohol-resistant foam, or water spray. 
Flash Point: 				Not applicable. 
Autoignition Temperature: 	Not available. 
Explosion Limits, Lower:		Not available. 
Upper: 				Not available. 
NFPA Rating: 	(estimated) Health: 3; Flammability: 0; Instability: 1

Section 6 - Accidental Release Measures

General Information: Use proper personal protective equipment as indicated in Section 8. 
Spills/Leaks: Vacuum or sweep up material and place into a suitable disposal container. Clean up spills immediately, observing precautions in the Protective Equipment section. Avoid generating dusty conditions. Cover with sand, dry lime or soda ash and place in a closed container for disposal. Provide ventilation.

Section 7 - Handling and Storage

Handling: Wash thoroughly after handling. Minimize dust generation and accumulation. Do not breathe dust, vapor, mist, or gas. Do not get in eyes, on skin, or on clothing. Keep container tightly closed. Use only in a chemical fume hood. Discard contaminated shoes. 
Storage: Store in a tightly closed container. Store in a cool, dry, well-ventilated area away from incompatible substances. Keep away from metals. Corrosives area. Do not store in metal containers.

Section 8 - Exposure Controls, Personal Protection

Engineering Controls: Facilities storing or utilizing this material should be equipped with an eyewash facility and a safety shower. Use adequate ventilation to keep airborne concentrations low. 
Exposure Limits

| Chemical Name | ACGIH | NIOSH | OSHA - Final PELs |
| --- | --- | --- | --- |
| Phosphorous acid | none listed | none listed | none listed |
| Phosphorous acid (two legit CAS numbers) | none listed | none listed | none listed |

OSHA Vacated PELs: Phosphorous acid: No OSHA Vacated PELs are listed for this chemical. Phosphorous acid (two legit CAS numbers): No OSHA Vacated PELs are listed for this chemical. 
Personal Protective Equipment 
Eyes: Wear chemical splash goggles. 
Skin: Wear appropriate protective gloves to prevent skin exposure. 
Clothing: Wear appropriate protective clothing to prevent skin exposure. 
Respirators: A respiratory protection program that meets OSHA's 29 CFR 1910.134 and ANSI Z88.2 requirements or European Standard EN 149 must be followed whenever workplace conditions warrant respirator use.

Section 9 - Physical and Chemical Properties

Physical State: 			Crystals 
Appearance: 			white or yellow 
Odor: 					none reported 
pH: 					Acidic in solution. 
Vapor Pressure: 			Not applicable. 
Vapor Density: 			Not available. 
Evaporation Rate:			Not applicable. 
Viscosity: 				Not applicable. 
Boiling Point: 			392 deg F 
Freezing/Melting Point:		165 deg F 
Decomposition Temperature:	Not available. 
Solubility: 				Soluble. 
Specific Gravity/Density:		1.7 
Molecular Formula:		H3PO3 
Molecular Weight:			82.0

Section 10 - Stability and Reactivity

Chemical Stability: Stable at room temperature in closed containers under normal storage and handling conditions. Hygroscopic: absorbs moisture or water from the air. Phosphorous acid, ortho reacts with air to form Phosphoric acid. 
Conditions to Avoid:   Dust generation, exposure to air, exposure to moist air or water. 
Incompatibilities with Other Materials:   Strong bases, strong oxidizing agents, strong reducing agents, water, metals. 
Hazardous Decomposition Products:   	Oxides of phosphorus, phosphine. 
Hazardous Polymerization:   		Has not been reported.

Section 11 - Toxicological Information

CAS# 13598-36-2: SZ6400000 
LD50/LC50:
Not available.

CAS# 13598-36-2:
     Oral, mouse: LD50 = 1700 mg/kg;
     Oral, rat: LD50 = 1895 mg/kg;

Carcinogenicity:
CAS# 10294-56-1: Not listed by ACGIH, IARC, NTP, or CA Prop 65.
CAS# 13598-36-2: Not listed by ACGIH, IARC, NTP, or CA Prop 65.

Epidemiology: 			No information found 
Teratogenicity: 			No information found 
Reproductive Effects: 		No information found 
Mutagenicity: 			No information found 
Neurotoxicity: 			No information found 
Other Studies: 			None known

Section 12 - Ecological Information

No information available.

Section 13 - Disposal Considerations

Chemical waste generators must determine whether a discarded chemical is classified as a hazardous waste. US EPA guidelines for the classification determination are listed in 40 CFR Parts 261.3. Additionally, waste generators must consult state and local hazardous waste regulations to ensure complete and accurate classification. 
RCRA P-Series: None listed. 
RCRA U-Series: None listed.

Section 14 - Transport Information

|  | US DOT | Canada TDG |
| --- | --- | --- |
| Shipping Name: | PHOSPHOROUS ACID | PHOSPHOROUS ACID |
| Hazard Class: | 8 | 8 |
| UN Number: | UN2834 | UN2834 |
| Packing Group: | III | III |

Section 15 - Regulatory Information

US FEDERAL

TSCA 
     CAS# 10294-56-1 is listed on the TSCA inventory. 
     CAS# 13598-36-2 is listed on the TSCA inventory. 
Health & Safety Reporting List
     None of the chemicals are on the Health & Safety Reporting List. 
Chemical Test Rules
     None of the chemicals in this product are under a Chemical Test Rule. 
Section 12b
     None of the chemicals are listed under TSCA Section 12b. 
TSCA Significant New Use Rule
     None of the chemicals in this material have a SNUR under TSCA. 
CERCLA Hazardous Substances and corresponding RQs
     None of the chemicals in this material have an RQ. 
SARA Section 302 Extremely Hazardous Substances
     None of the chemicals in this product have a TPQ. 
SARA Codes
     CAS # 10294-56-1: delayed, reactive. 
Section 313      No chemicals are reportable under Section 313. 
Clean Air Act:
     This material does not contain any hazardous air pollutants. 
     This material does not contain any Class 1 Ozone depletors. 
     This material does not contain any Class 2 Ozone depletors. 
Clean Water Act:
     None of the chemicals in this product are listed as Hazardous Substances under the CWA. 
     None of the chemicals in this product are listed as Priority Pollutants under the CWA. 
     None of the chemicals in this product are listed as Toxic Pollutants under the CWA. 
OSHA:
     None of the chemicals in this product are considered highly hazardous by OSHA. 
STATE
     CAS# 10294-56-1 is not present on state lists from CA, PA, MN, MA, FL, or NJ. 
     CAS# 13598-36-2 can be found on the following state right to know lists: New Jersey. 

California Prop 65
California No Significant Risk Level: None of the chemicals in this product are listed. 

European/International Regulations
European Labeling in Accordance with EC Directives
Hazard Symbols:
     C 
Risk Phrases:
     R 22 Harmful if swallowed. 
     R 35 Causes severe burns. 

Safety Phrases:
     S 26 In case of contact with eyes, rinse immediately with plenty of 
     water and seek medical advice. 
     S 36/37/39 Wear suitable protective clothing, gloves and eye/face pr 
     otection. 
     S 45 In case of accident or if you feel unwell, seek medical advice 
     immediately (show the label where possible). 

WGK (Water Danger/Protection)
     CAS# 10294-56-1: No information available. 
     CAS# 13598-36-2: 1 
Canada - DSL/NDSL
     CAS# 10294-56-1 is listed on Canada's DSL List. 
     CAS# 13598-36-2 is listed on Canada's DSL List. 
Canada - WHMIS
     This product has a WHMIS classification of E. 
This product has been classified in accordance with the hazard criteria of the Controlled Products Regulations and the MSDS contains all of the information required by those regulations. 
Canadian Ingredient Disclosure List
     CAS# 13598-36-2 is listed on the Canadian Ingredient Disclosure List.

Section 16 - Additional Information

MSDS Creation Date: 12/12/1997 
Revision #7 Date: 1/15/2004 

The information above is believed to be accurate and represents the best information currently available to us. However, we make no warranty of merchantability or any other warranty, express or implied, with respect to such information, and we assume no liability resulting from its use. Users should make their own investigations to determine the suitability of the information for their particular purposes. In no event shall Fisher be liable for any claims, losses, or damages of any third party or for lost profits or any special, indirect, incidental, consequential or exemplary damages, howsoever arising, even if Fisher has been advised of the possibility of such damages.