* * * * * Effective Date: 11/09/08 * * * * *

# ISOBUTYRIC ACID

## 1. Product Identification

Synonyms:   Dimethylacetic Acid, 2-Methylpropionic Acid, 2-Methylpropanoic Acid 
CAS No.:      79-31-2 
Chemical Formula:    (CH3)2CHCO2H

## 2. Composition/Information on Ingredients

Ingredient                    CAS No         Percent        Hazardous

------------------------   ------------   ------------      ---------

Isobutyric Acid            79-31-2          100%             Yes

## 3. Hazards Identification

Emergency Overview 
-------------------------- 
DANGER! CORROSIVE.  HARMFUL IF SWALLOWED OR INHALED. CAUSES SEVERE IRRITATION AND BURNS TO EVERY AREA OF CONTACT.  READILY ABSORBED THROUGH SKIN.  COMBUSTIBLE.

SAF-T-DATA(tm) Ratings (Provided here for your convenience) 
----------------------------------------------------------------------------------------------------------- 
Health Rating: 		3 - Severe 
Flammability Rating: 		2 - Moderate 
Reactivity Rating: 		0 - Slight

Lab Protective Equip: GOGGLES & SHIELD; LAB COAT & APRON; VENT HOOD; PROPER GLOVES 
Storage Color Code: White (Corrosive) 
----------------------------------------------------------------------------------------------------------- 

Potential Health Effects 
---------------------------------- 
Information on the human health effects from exposure to this substance is limited. 
Inhalation: 
Inhalation is irritating to the mucous membrane and upper respiratory tract. In extreme cases, severe irritation and burns may result. 
Ingestion: 
Material causes irritation and burns to digestive tract. 
Skin Contact: 
Causes irritation and burns to the skin. 
Eye Contact: 
Can cause severe irritation or burns with eye damage. 
Chronic Exposure: 
No information found. 
Aggravation of Pre-existing Conditions: 
No information found.

## 4. First Aid Measures

Inhalation: 
Remove to fresh air. If not breathing, give artificial respiration. If breathing is difficult, give oxygen. Call a physician. 
Ingestion: 
DO NOT INDUCE VOMITING. Give large quantities of water. Never give anything by mouth to an unconscious person. Call a physician immediately. 
Skin Contact: 
In case of contact, immediately flush skin with plenty of water for at least 15 minutes while removing contaminated clothing and shoes. Wash clothing before reuse. Call a physician. 
Eye Contact: 
Immediately flush eyes with plenty of water for at least 15 minutes, lifting lower and upper eyelids occasionally. Get medical attention immediately.

## 5. Fire Fighting Measures

Fire: 
Combustible liquid.   Autoignition Temperature = 440ºC
Explosion: 
Flash Point = 131ºF (55ºC)			Method:  Closed Cup 
Fire Extinguishing Media: 
Water spray, dry chemical, alcohol foam, or carbon dioxide.

Special Information: 
In the event of a fire, wear full protective clothing and NIOSH-approved self-contained breathing apparatus with full facepiece operated in the pressure demand or other positive pressure mode.

## 6. Accidental Release Measures

Ventilate area of leak or spill. Wear appropriate personal protective equipment as specified in Section 8. Isolate hazard area.  Keep unnecessary and unprotected personnel from entering.  Contain and recover liquid when possible.

Neutralize with alkaline material (soda ash, lime), then absorb with an inert material (e. g., vermiculite, dry sand, earth), and place in a chemical waste container.  Do not use combustible materials, such as saw dust. Do not flush to sewer!

## 7. Handling and Storage

Keep in a tightly closed container, stored in a cool, dry, ventilated area.  Protect against physical damage. Isolate from incompatible substances.

Containers of this material may be hazardous when empty since they retain product residues (vapors, liquid); observe all warnings and precautions listed for the product.

## 8. Exposure Controls/Personal Protection

Ventilation System: 
A system of local and/or general exhaust is recommended to keep employee exposures as low as possible.  Local exhaust ventilation is generally preferred because it can control the emissions of the contaminant at its source, preventing dispersion of it into the general work area.

Personal Respirators (NIOSH Approved): 
For conditions of use where exposure to the substance is apparent and engineering controls are not feasible, consult an industrial hygienist.  For emergencies, or instances where the exposure levels are not known, use a full-facepiece positive-pressure, air-supplied respirator.

Skin Protection: 
Wear impervious protective clothing, including boots, gloves, lab coat, apron or coveralls, as appropriate, to prevent skin contact. 
Eye Protection: 
Use chemical safety goggles and/or full face shield where dusting or splashing of solutions is possible. Maintain eye wash fountain and quick-drench facilities in work area.

General Hygiene Measures: 
Wash thoroughly after handling.

## 9. Physical and Chemical Properties

Appearance: 					Clear liquid 
Odor: 						Pungent 
Solubility: 					Soluble in water, chloroform, ether, alcohol. 
Specific Gravity: 				0.949 
pH: 						2.5 
% Volatiles by volume @ 21C (70F): 	No information found. 
Boiling Point: 				154ºC 
Melting Point: 				-47ºC 
Vapor Density (Air=1): 			3.04 g/l 
Vapor Pressure (mm Hg): 			1.5 mmHg 
Evaporation Rate (BuAc=1): 		No information found.

Viscosity:					1.13 Pas

Flash Point:					131ºF

Refractive Index:				1.393

## 10. Stability and Reactivity

Stability: 
Stable under ordinary conditions of use and storage. 
Hazardous Decomposition Products: 
Carbon dioxide and carbon monoxide may form when heated to decomposition.  
Hazardous Polymerization: 
Will not occur. 
Incompatibilities: 
Strong oxidizers. 
Conditions to Avoid: 
Incompatibles.

## 11. Toxicological Information

Skin Contact:  May cause skin irritation.

Skin Absorption:  Harmful if absorbed through skin.

Eye Contact:  May cause eye irritation.

Inhalation:  May be harmful if inhaled.  May be irritating to mucous membranes and upper respiratory tract.

Ingestion:  Harmful if swallowed.

## 12. Ecological Information

Environmental Fate: 		No information found. 
Environmental Toxicity: 		No information found.

## 13. Disposal Considerations

Whatever cannot be saved for recovery or recycling should be handled as hazardous waste and sent to a RCRA approved incinerator or disposed in a RCRA approved waste facility.  Processing, use or contamination of this product may change the waste management options.  State and local disposal regulations may differ from federal disposal regulations. Dispose of container and unused contents in accordance with federal, state and local requirements.

## 14. Transport Information

Domestic (Land, D.O.T.) 
----------------------- 
Proper Shipping Name:   Isobutyric Acid 
Hazard Class: 3  (flammable liquid) 
UN/NA: UN2529 
Packing Group: III
Information reported for product/size: 500ML 

International (Water, I.M.O.) 
----------------------------- 
Proper Shipping Name:   Isobutyric Acid 
Hazard Class: 3 
UN/NA: UN2529 
Packing Group: III 
Information reported for product/size: 500ML 

International (Air, I.C.A.O.) 
----------------------------- 
Proper Shipping Name: Isobutyric Acid 
Hazard Class:   3 
UN/NA: UN2529 
Packing Group: III 
Information reported for product/size: 500ML

## 15. Regulatory Information

-\Chemical Inventory Status - Part 1\---------------------------------

Ingredient                              TSCA  EC   Japan  Australia

--------------------------------------  ----  ---  -----  ---------

Isobutyric Acid(79-31-2)              Yes   Yes   Yes      Yes

-\Chemical Inventory Status - Part 2\---------------------------------

--Canada--

Ingredient                              Korea  DSL   NDSL  Phil.

--------------------------------------  -----  ---   ----  -----

Isobutyric Acid (79-31-2)              Yes   Yes   No     Yes

-\Federal, State & International Regulations - Part 1\----------------

-SARA 302-    ------SARA 313------

Ingredient                         RQ    TPQ     List  Chemical Catg.

----------------------------------  ---   -----   ----  --------------

Isobutyric Acid (79-31-2)          No     No      No         No

-\Federal, State & International Regulations - Part 2\----------------

-RCRA-    -TSCA-

Ingredient                              CERCLA     261.33     8(d)

--------------------------------------  ------     ------    ------

Isobutyric Acid (79-31-2)                  No        No        No

Chemical Weapons Convention:  No     TSCA 12(b):  No     CDTA:  No

SARA 311/312:  Acute: Yes      Chronic: No   Fire: Yes  Pressure: No

Reactivity: No

Australian Hazchem Code: None allocated. 
Poison Schedule: None allocated. 
WHMIS: 
This MSDS has been prepared according to the hazard criteria of the Controlled Products Regulations (CPR) and the MSDS contains all of the information required by the CPR.

## 16. Other Information

NFPA Ratings: 	Health: 3 	Flammability: 2 	Reactivity: 0 
Label Hazard Warning: 
DANGER! CORROSIVE. HARMFUL IF SWALLOWED OR INHALED. CAUSES SEVERE IRRITATION AND BURNS TO EVERY AREA OF CONTACT. 
Label Precautions: 
Do not breathe vapor or mist.
Do not get in eyes, on skin, or on clothing.
Keep container closed.
Use only with adequate ventilation.
Wash thoroughly after handling. 
Label First Aid: 
In case of contact, immediately flush eyes or skin with plenty of water for at least 15 minutes. Remove contaminated clothing and shoes. Wash clothing before reuse. Call a physician. If inhaled, remove to fresh air. If not breathing, give artificial respiration. If breathing is difficult, give oxygen. Call a physician. If swallowed, DO NOT INDUCE VOMITING. Give large quantities of water. Never give anything by mouth to an unconscious person. Call a physician immediately. 
Product Use: 
Laboratory Reagent. 
Revision Information: 
MSDS Section(s) changed since last revision of document include: 3. 
Disclaimer: 
************************************************************************ 
The information contained herein is provided in good faith but makes no representation as to its comprehensiveness or accuracy.  This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product.  Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

DGR INDUSTRIAL PRODUCTS, INC. MAKES NO REPRESENTATIONS OR WARRANTIES, EITHER EXPRESS OR IMPLIED, INCLUDING WITHOUT LIMITATION ANY WARRANTIES OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE WITH RESPECT TO THE INFORMATION SET FORTH HEREIN OR THE PRODUCT TO WHICH THE INFORMATION REFERS.  ACCORDINGLY, DGR INDUSTRIAL PRODUCTS, INC. WILL NOT BE RESPONSIBLE FOR DAMAGES RESULTING FROM USE OF OR RELIANCE UPON THIS INFORMATION. 
************************************************************************