MSDS Number: S3360 * * * * * Effective Date: 11/10/05 * * * * * Supercedes: 02/18/03

# Sodium Chlorite, Anhydrous, Flakes

## 1. Product Identification

Synonyms: None 
CAS No.: 7758-19-2 
Molecular Weight: 90.44 
Chemical Formula: NaClO2 
Product Codes: V044

## 2. Composition/Information on Ingredients

Ingredient                     CAS No         Percent        Hazardous

--------------------------   ------------   ------------     ---------

Sodium Chlorite               7758-19-2       79 - 100%        Yes

## 3. Hazards Identification

Emergency Overview 
-------------------------- 
DANGER! CAUSES SEVERE SKIN AND EYE BURNS. STRONG OXIDIZER. CONTACT WITH OTHER MATERIAL MAY CAUSE FIRE. CAN VIOLENTLY DECOMPOSE AT ELEVATED TEMPERATURES. HARMFUL IF INHALED OR SWALLOWED CAUSES EYE AND SKIN IRRITATION. 

Potential Health Effects 
---------------------------------- 

Inhalation: 
Is harmful 
Ingestion: 
Is harmful 
Skin Contact: 
Severe irritation or burns. 
Eye Contact: 
Severe irritation or burns. 
Chronic Exposure: 
No information found. 
Aggravation of Pre-existing Conditions: 
Damaged skin.

## 4. First Aid Measures

Inhalation: 
If inhaled, remove to fresh air. If not breathing, give artificial respiration. If breathing is difficult, give oxygen. Prompt action is essential. 
Ingestion: 
If swallowed, DO NOT INDUCE VOMITING. Give large quantities of water. Never give anything by mouth to an unconscious person. Get medical attention immediately. 
Skin Contact: 
In case of contact, immediately flush skin with plenty of water for at least 15 minutes while removing contaminated clothing and shoes. Wash clothing before re-use. 
Eye Contact: 
In case of eye contact, immediately flush with plenty of water for at least 15 minutes.

## 5. Fire Fighting Measures

Fire: 
Not expected to be a fire hazard. 
Explosion: 
Contact with acids may generate spontaneously flammable chlorine dioxide gas. Greatly increases the burning rate of combustible materials. Fire or excessive heat may produce hazardous decomposition products. This material in sufficient quantity and reduced particle size is capable of creating a dust explosion 
Fire Extinguishing Media: 
Use extinguishing media appropriate for surrounding fire. 
Special Information: 
Wear self-contained breathing apparatus and protective clothing to prevent contact with skin and eyes. Firefighters should wear proper protective equipment and self-contained breathing apparatus with full facepiece operated in positive pressure mode. Move exposed containers from fire area if it can be done without risk. Use water to keep fire-exposed containers cool. Fight fire from protected location

## 6. Accidental Release Measures

Flush to sewer with large amounts of water.

## 7. Handling and Storage

Material is a strong oxidizer. Keep from contact with clothing and other combustible materials. Remove and wash contaminated clothing promptly. Do not store near combustible materials. Store in tightly closed container. Since empty container may retain product residue, follow label warnings even after container is empty. Keep away from fire. Fire or high temperatures may cause explosive decomposition.

## 8. Exposure Controls/Personal Protection

Airborne Exposure Limits: 
None established. 
Ventilation System: 
A system of local and/or general exhaust is recommended to keep employee exposures as low as possible. Local exhaust ventilation is generally preferred because it can control the emissions of the contaminant at its source, preventing dispersion of it into the general work area. Please refer to the ACGIH document, Industrial Ventilation, A Manual of Recommended Practices, most recent edition, for details. 
Personal Respirators (NIOSH Approved): 
For conditions of use where exposure to the substance is apparent and engineering controls are not feasble, consult an industrial hygienist. For emergencies, or instances where the exposure levels are not known, use a full-facepiece positive-pressure, air-supplied respirator. WARNING: Air purifying respirators do not protect workers in oxygen-deficient atmospheres. 
Skin Protection: 
Wear impervious protective clothing, including boots, gloves, lab coat, apron or coveralls, as appropriate, to prevent skin contact. 
Eye Protection: 
Use chemical safety goggles and/or full face shield where dusting or splashing of solutions is possible. Maintain eye wash fountain and quick-drench facilities in work area. 
Other Control Measures: 
There is insufficient data in the published literature to assign complete numerical SAF-T-DATA* ratings and laboratory protective equipment for this product. Special precautions must be used in storage, use and handling. Protective equipment for laboratory bench use should be chosen using professional judgment based on the size and type of reaction or test to be conducted and the available ventilation, with overriding consideration to minimize contact with the chemical.

## 9. Physical and Chemical Properties

Appearance: 
White Crystalline solid. 
Odor: 
No information found. 
Solubility: 
Appreciable (> 10%) 
Specific Gravity: 
No information found. 
pH: 
No information found. 
% Volatiles by volume @ 21C (70F): 
0 
Boiling Point: 
No information found. 
Melting Point: 
No information found. 
Vapor Density (Air=1): 
No information found. 
Vapor Pressure (mm Hg): 
0 @ 20C (68F) 
Evaporation Rate (BuAc=1): 
0

## 10. Stability and Reactivity

Stability: 
Stable under ordinary conditions of use and storage. 
Hazardous Decomposition Products: 
Hydrogen chloride gas 
Hazardous Polymerization: 
Will not occur. 
Incompatibilities: 
Acids, Reducing agents combustible materials. 
Conditions to Avoid: 
Avoid temperatures above 60C Avoid temperatures above 280C

## 11. Toxicological Information

-\Cancer Lists\------------------------------------------------------

---NTP Carcinogen---

Ingredient                       Known    Anticipated    IARC Category

------------------------------   -----    -----------    -------------

Sodium Chlorite (7758-19-2)        No          No              3

## 12. Ecological Information

Environmental Fate: 
No information found. 
Environmental Toxicity: 
No information found.

## 13. Disposal Considerations

Whatever cannot be saved for recovery or recycling should be handled as hazardous waste and sent to a RCRA approved waste facility. Processing, use or contamination of this product may change the waste management options. State and local disposal regulations may differ from federal disposal regulations. Dispose of container and unused contents in accordance with federal, state and local requirements.

## 14. Transport Information

Domestic (Land, D.O.T.) 
----------------------- 
Proper Shipping Name: SODIUM CHLORITE 
Hazard Class: 5.1 
UN/NA: UN1496 
Packing Group: II 
Information reported for product/size: 500G 

International (Water, I.M.O.) 
----------------------------- 
Proper Shipping Name: SODIUM CHLORITE 
Hazard Class: 5.1 
UN/NA: UN1496 
Packing Group: II 
Information reported for product/size: 500G

## 15. Regulatory Information

-\Chemical Inventory Status - Part 1\---------------------------------

Ingredient                                 TSCA  EC   Japan  Australia

-----------------------------------------  ----  ---  -----  ---------

Sodium Chlorite (7758-19-2)                 Yes  Yes   Yes      Yes

-\Chemical Inventory Status - Part 2\---------------------------------

--Canada--

Ingredient                                    Korea  DSL   NDSL  Phil.

---------------------------------------------  -----  ---   ----  -----

Sodium Chlorite (7758-19-2)                     Yes   Yes   No     Yes

-\Federal, State & International Regulations - Part 1\----------------

-SARA 302-    ------SARA 313------

Ingredient                           RQ    TPQ     List  Chemical Catg.

----------------------------------  ---   -----   ----  --------------

Sodium Chlorite (7758-19-2)          No    No      No         No

-\Federal, State & International Regulations - Part 2\----------------

-RCRA-    -TSCA-

Ingredient                                 CERCLA     261.33     8(d)

-----------------------------------------  ------     ------    ------

Sodium Chlorite (7758-19-2)                  No         No         No

Chemical Weapons Convention:  No     TSCA 12(b):  No     CDTA:  No

SARA 311/312:  Acute: Yes      Chronic: Yes  Fire: Yes Pressure: No

Reactivity: No          (Pure / Solid)

Australian Hazchem Code: 2R 
Poison Schedule: None allocated. 
WHMIS: 
This MSDS has been prepared according to the hazard criteria of the Controlled Products Regulations (CPR) and the MSDS contains all of the information required by the CPR.

## 16. Other Information

Label Hazard Warning: 
DANGER! CAUSES SEVERE SKIN AND EYE BURNS. STRONG OXIDIZER. CONTACT WITH OTHER MATERIAL MAY CAUSE FIRE. CAN VIOLENTLY DECOMPOSE AT ELEVATED TEMPERATURES. HARMFUL IF INHALED OR SWALLOWED CAUSES EYE AND SKIN IRRITATION. 
Label Precautions: 
No SAF-T-DATA Ratings have been developed for this product. Read and follow all warnings, precautions, instructions and other safety and handling information on the label and MSDS.
Avoid contact with eyes, skin, clothing.
Avoid breathing dust. Wash thoroughly after handling. Use with adequate ventilation. Remove and wash contaminated clothing promptly. In case of fire, soak with water. In case of spill, carefully sweep up and remove. Flush area with water. 
Label First Aid: 
If swallowed, DO NOT INDUCE VOMITING. Give large quantities of water. Never give anything by mouth to an unconscious person. If inhaled, remove to fresh air. If not breathing, give artificial respiration. If breathing is difficult, give oxygen. Prompt action is essential. In case of contact, immediately flush eyes or skin with plenty of water for at least 15 minutes while removing contaminated clothing and shoes. Wash clothing before reuse. 
Product Use: 
Laboratory Reagent. 
Revision Information: 
No Changes. 
Disclaimer: 
************************************************************************ 
Mallinckrodt Baker, Inc. provides the information contained herein in good faith but makes no representation as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose. MALLINCKRODT BAKER, INC. MAKES NO REPRESENTATIONS OR WARRANTIES, EITHER EXPRESS OR IMPLIED, INCLUDING WITHOUT LIMITATION ANY WARRANTIES OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE WITH RESPECT TO THE INFORMATION SET FORTH HEREIN OR THE PRODUCT TO WHICH THE INFORMATION REFERS. ACCORDINGLY, MALLINCKRODT BAKER, INC. WILL NOT BE RESPONSIBLE FOR DAMAGES RESULTING FROM USE OF OR RELIANCE UPON THIS INFORMATION. 
************************************************************************ 
Prepared by: Environmental Health & Safety
Phone Number: (314) 654-1600 (U.S.A.)