Material Safety Data Sheet

AMMONIUM PERCHLORATE

=======================================================================

Ingredients/Identity Information

=======================================================================

Proprietary: NO

Ingredient: AMMONIUM PERCHLORATE

Ingredient Sequence Number: 01

Percent: 99.5MIN

NIOSH (RTECS) Number: SC7520000

CAS Number: 7790-98-9

OSHA PEL: NOT ESTABLISHED

ACGIH TLV: NOT ESTABLISHED

Other Recommended Limit: 5MG/CUM,AS RESP DUST

=======================================================================

Physical/Chemical Characteristics

=======================================================================

Appearance And Odor: ODORLESS WHITE GRANULAR CRYSTALS.

Melting Point: 842F/450C

Specific Gravity: 1.95

Decomposition Temperature: >150F,>66C

Solubility In Water: APPRECIABLE

Magnetism (Milligauss): N/P

Corrosion Rate (IPY): UNKNOWN

=======================================================================

Fire and Explosion Hazard Data

=======================================================================

Flash Point Method: N/P

Extinguishing Media: WATER SPRAY, FOAM.

Special Fire Fighting Proc: USE NIOSH/MSHA APPROVED SCBA IN AN ENCLOSED

AREA.

Unusual Fire And Expl Hazrds: COOL W/WATER FOR LARGE FIRE IN STORAGE AREA,

USE UNMANNED HOSE HOLDER OR WITHDRAW & LET BURN! SEE SUP

=======================================================================

Reactivity Data

=======================================================================

Stability: NO

Cond To Avoid (Stability): STABLE BELOW 150 F.DECOMP OR EXPLODES AT HIGHER

TEMP.

Materials To Avoid: COMBUSTIBLES,OXIDIZERS CREATE EXTREME FIRE/EXPL

HAZARD.

Hazardous Decomp Products: VAPORS OF CHLORINE,HYDROGEN CHLORIDES,

AMMONIA,OXIDES OF NITROGEN.

Hazardous Poly Occur: NO

=======================================================================

Health Hazard Data

=======================================================================

LD50-LC50 Mixture: 4200 MG/KG(RAT) & 1900MG/KG (RABBIT)

Route Of Entry - Inhalation: YES

Route Of Entry - Skin: YES

Route Of Entry - Ingestion: YES

Health Haz Acute And Chronic: ACUTE:MILD IRRITANT TO THE SKIN,EYES,MUCOUS

MEMBRANES,RESPIRATORY OR G.I. TRACT,REDNESS OF EYES AND SKIN. CHRONIC:SKIN,

G.I. OR RESPIRATORY DISORDERS.

Carcinogenicity - NTP: NO

Carcinogenicity - IARC: NO

Carcinogenicity - OSHA: NO

Explanation Carcinogenicity: PER MSDS DATA.

Signs/Symptoms Of Overexp: EYE:IRRT,REDNESS.SKIN:IRRT,REDNESS.INHL:URT

IRRT.INGEST: GI TRACT IRRT,DIARR.

Med Cond Aggravated By Exp: PRE-EXISTING CONDITIONS MAY BE WORSEN.

Emergency/First Aid Proc: INHAL:RMV TO FRESH AIR. IF NOT BRTHNG GIVE CPR;

IF BRTHNG DIFF GIVE OXYGEN. EYE:IMMED FLUSH W/PLENTY OF WATER. SKIN: WASH

W/SOAP&WATER. RMV CONTAM CLTHG&SHOES. INGEST:INDUCE VOMIT. RPT UNTIL VOMIT

IS CLEAR. NOTHG BY MOUTH IF UNCONSC. GET MEDICAL ATTN.

=======================================================================

Precautions for Safe Handling and Use

=======================================================================

Steps If Material Released/Spill: SCOOP UP OR ABSORB WITH NONCOMBUSTIBLE ABSORBENT(VERMICULITE).

Waste Disposal Method: EPA HAZWASTE D001. DISPOSE AS HAZWASTE IAW ALL LAWS & REGS.

Precautions-Handling/Storing: DO NOT ALLOW CONTACT W/COMBUSTIBLE

MATERIALS. STORE IN ORIGINAL CLOSED CONTAINERS. HANDLE CONTAINERS CAREFULLY. AVOID INHL DUST & EYE/SKN CONTACT.

Other Precautions: PROVIDE WATER FILLED JUMP TANK OR SAFETY SHOWER. WASH CONTAMINATED CLOTHING PROMPTLY

=======================================================================

Control Measures

=======================================================================

Respiratory Protection: USE NIOSH/MSHA APPROVED RESPIRATOR FOR DUST IF

ABOVE PEL/TLV.

Ventilation: LOCAL OR GENERAL AS REQD.

Protective Gloves: IMPERVIOUS

Eye Protection: SAFETY GLASSES OR GOGGLES

Other Protective Equipment: WEAR PROTECTIVE CLOTHING, RUBBER BOOTS. DO NOT

WEAR LEATHER.

Work Hygienic Practices: AVOID CONTACT WITH EYES AND SKIN;DO NOT BREATHE

DUST/MIST;KEEP FROM CONTACT WITH CLOTHING & OTHER COMBUSTIBLE MATERIAL.

Suppl. Safety & Health Data: .

=======================================================================

Transportation Data

=======================================================================

Trans Data Review Date:   87075

DOT PSN Code: ASD

DOT Proper Shipping Name: AMMONIUM PERCHLORATE

DOT Class: 5.1

DOT ID Number: UN1442

DOT Pack Group: II

DOT Label: OXIDIZER

IMO PSN Code: AZZ

IMO Proper Shipping Name: AMMONIUM PERCHLORATE

IMO Regulations Page Number: 5126

IMO UN Number: 1442

IMO UN Class: 5.1

IATA PSN Code: BQC

IATA UN ID Number: 1442

IATA Proper Shipping Name: AMMONIUM PERCHLORATE

IATA UN Class: 5.1

IATA Label: OXIDIZER

AFI PSN Code: BQC

AFI Prop. Shipping Name: AMMONIUM PERCHLORATE

AFI Class: 5.1

AFI ID Number: UN1442

AFI Pack Group: II

AFI Label: OXIDIZER

AFI Special Prov: A9

AFI Basic Pac Ref: 9-10

=======================================================================

Disposal Data

=======================================================================

Disposal Data Review Date: 88229

Landfill Ban Item: YES

Disposal Supplemental Data: MSDS DTD JUN85.FIRE(CONT):EVACUATE AREA,DENY

ENTRY.USE SCBA PP MODE, FULL PROTECTIVE CLOTHING. IN CASE OF ACCIDENTAL

EXPOSURE OR DISCHARGE, CONSULT HEALTH AND SAFETY FILE FOR PRECAUTIONS.

1st EPA Haz Wst Code New: D001

1st EPA Haz Wst Name New: IGNITIBLE

1st EPA Haz Wst Char New: IGNITABILITY

1st EPA Acute Hazard New: NO

2nd EPA Haz Wst Code New: D003

2nd EPA Haz Wst Name New: REACTIVE

2nd EPA Haz Wst Char New: REACTIVITY

=======================================================================

Label Data

=======================================================================

Common Name: AMMONIUM PERCHLORATE

Chronic Hazard: YES

Signal Word: DANGER!

Acute Health Hazard-Slight: 1

Contact Hazard-Slight: 1

Fire Hazard-None: 0

Reactivity Hazard-Severe: 4

Special Hazard Precautions: ACUTE-MINOR IRRITANT TO EYE,SKIN,RESPIRATORY &

GI TRACT.CHRONIC-SKIN,GI OR RESPIRATORY DISORDERS.STORAGE-DANGER!STRONG

OXIDIZER-CONTACT WITH COMBUSTIBLE OR OXIDIZABLE MATERIAL CONSTITUTES AN

EXTREME FIRE & EXPLOSION HAZARD. CONTAMINATED CLOTHING & ORGANIC METERIALS

ARE DANGEROUSLY FLAMMABLE. ALWAYS HAVE A WATER FILLED JUMP TANK OR DELUGE

SHOWER NEARBY.IF YOUR CLOTHING CATCHES FIRE,DON'T USE A FIRE BLANKET. USE THE JUMP TANK OR DELUGE SHOWER. DON'T WEAR LEATHER MATERIALS. STORE

CONTAINERS TIGHTLY CLOSED. FIRST AID-CALL A DOCTOR.

EYE:FLUSH W/WATER FOR 15

INDUCE VOMITING. GIVE WATER.

Protect Eye: Y

Protect Skin: Y

Protect Respiratory: Y