MSDS Number: B6184 * * * * * Effective Date: 09/16/09 * * * * * Supersedes: 08/02/07

# N-BUTYL ACETATE

## 1. Product Identification

Synonyms: 1-Butyl acetate; butyl ethanoate; acetic acid, n-butyl ester 
CAS No.: 123-86-4 
Molecular Weight: 116.16 
Chemical Formula: C6H12O2 
Product Codes: 
J.T. Baker: 5365, 5681, 9159, 9161, 9164, 9171, 9173, 9191, D683 
Mallinckrodt: 1820, H984

## 2. Composition/Information on Ingredients

Ingredient                   CAS No         Percent      Hazardous

------------------------   ------------   ------------   ---------

n-Butyl Acetate             123-86-4        90 - 100%       Yes

## 3. Hazards Identification

Emergency Overview 
-------------------------- 
WARNING! FLAMMABLE LIQUID AND VAPOR. HARMFUL IF SWALLOWED OR INHALED. CAUSES SEVERE IRRITATION TO EYES. CAUSES IRRITATION TO SKIN AND RESPIRATORY TRACT. AFFECTS CENTRAL NERVOUS SYSTEM. 

SAF-T-DATA(tm) Ratings (Provided here for your convenience) 
----------------------------------------------------------------------------------------------------------- 
Health Rating: 2 - Moderate (Life) 
Flammability Rating: 2 - Moderate 
Reactivity Rating: 1 - Slight 
Contact Rating: 3 - Severe (Life) 
Lab Protective Equip: GOGGLES & SHIELD; LAB COAT & APRON; VENT HOOD; PROPER GLOVES; CLASS B EXTINGUISHER 
Storage Color Code: Red (Flammable) 
----------------------------------------------------------------------------------------------------------- 

Potential Health Effects 
---------------------------------- 

Inhalation: 
Causes irritation to the respiratory tract. Symptoms may include coughing, shortness of breath. High concentrations have a narcotic effect. 
Ingestion: 
Irritant to tissues. Sore throat, abdominal pain, nausea, vomiting, diarrhea are the symptoms. Expected to have a narcotic effect. One ounce may produce severe poisoning. 
Skin Contact: 
This material degreases the skin. Irritation and discoloration of the skin are symptoms. Skin allergy occasionally develops. Persons who have become allergic can develop rash upon future exposure to low levels. 
Eye Contact: 
Vapors cause eye irritation. Splashes cause severe irritation, possible corneal burns and eye damage. 
Chronic Exposure: 
Repeated or prolonged skin contact may defat the skin and produce irritation and dermatitis. Kidney and liver damage are reported in animals. 
Aggravation of Pre-existing Conditions: 
Persons with pre-existing skin disorders or eye problems, or impaired liver, kidney or respiratory function may be more susceptible to the effects of the substance.

## 4. First Aid Measures

Inhalation: 
Remove to fresh air. If not breathing, give artificial respiration. If breathing is difficult, give oxygen. Get medical attention. 
Ingestion: 
Induce vomiting immediately as directed by medical personnel. Never give anything by mouth to an unconscious person. Get medical attention. 
Skin Contact: 
Immediately flush skin with plenty of soap and water for at least 15 minutes. Remove contaminated clothing and shoes. Get medical attention. Wash clothing before reuse. Thoroughly clean shoes before reuse. 
Eye Contact: 
Immediately flush eyes with plenty of water for at least 15 minutes, lifting lower and upper eyelids occasionally. Get medical attention immediately.

## 5. Fire Fighting Measures

Fire: 
Flash point: 26C (79F) CC
Autoignition temperature: 425C (797F)
Flammable limits in air % by volume: 
lel: 1.7; uel: 7.6 
Explosion: 
Above flash point, vapor-air mixtures are explosive within flammable limits noted above. Vapors can flow along surfaces to distant ignition source and flash back. Sensitive to static discharge. 
Fire Extinguishing Media: 
Dry chemical, alcohol foam or carbon dioxide. Water may be ineffective. Water spray may be used to keep fire exposed containers cool. 
Special Information: 
In the event of a fire, wear full protective clothing and NIOSH-approved self-contained breathing apparatus with full facepiece operated in the pressure demand or other positive pressure mode.

## 6. Accidental Release Measures

Ventilate area of leak or spill. Remove all sources of ignition. Wear appropriate personal protective equipment as specified in Section 8. Isolate hazard area. Keep unnecessary and unprotected personnel from entering. Contain and recover liquid when possible. Use non-sparking tools and equipment. Collect liquid in an appropriate container or absorb with an inert material (e. g., vermiculite, dry sand, earth), and place in a chemical waste container. Do not use combustible materials, such as saw dust. Do not flush to sewer! US Regulations (CERCLA) require reporting spills and releases to soil, water and air in excess of reportable quantities. The toll free number for the US Coast Guard National Response Center is (800) 424-8802. If a leak or spill has not ignited, use water spray to disperse the vapors, to protect personnel attempting to stop leak, and to flush spills away from exposures.

J. T. Baker SOLUSORB® solvent adsorbent is recommended for spills of this product.

## 7. Handling and Storage

Protect against physical damage. Store in a cool, dry well-ventilated location, away from any area where the fire hazard may be acute. Outside or detached storage is preferred. Separate from incompatibles. Containers should be bonded and grounded for transfers to avoid static sparks. Storage and use areas should be No Smoking areas. Use non-sparking type tools and equipment, including explosion proof ventilation. Do Not attempt to clean empty containers since residue is difficult to remove. Do not pressurize, cut, weld, braze, solder, drill, grind or expose such containers to heat, sparks, flame, static electricity or other sources of ignition: they may explode and cause injury or death. Containers of this material may be hazardous when empty since they retain product residues (vapors, liquid); observe all warnings and precautions listed for the product.

## 8. Exposure Controls/Personal Protection

Airborne Exposure Limits: 
-OSHA Permissible Exposure Limit (PEL):
150 ppm (TWA)
- ACGIH Threshold Limit Value (TLV)
150 ppm (TWA), 200 ppm (STEL) 
Ventilation System: 
A system of local and/or general exhaust is recommended to keep employee exposures below the Airborne Exposure Limits. Local exhaust ventilation is generally preferred because it can control the emissions of the contaminant at its source, preventing dispersion of it into the general work area. Please refer to the ACGIH document, Industrial Ventilation, A Manual of Recommended Practices, most recent edition, for details. Use explosion-proof equipment. 
Personal Respirators (NIOSH Approved): 
If the exposure limit is exceeded, a half-face organic vapor respirator may be worn for up to ten times the exposure limit or the maximum use concentration specified by the appropriate regulatory agency or respirator supplier, whichever is lowest. A full-face piece organic vapor respirator may be worn up to 50 times the exposure limit or the maximum use concentration specified by the appropriate regulatory agency or respirator supplier, whichever is lowest. For emergencies or instances where the exposure levels are not known, use a full-face piece positive-pressure, air-supplied respirator. WARNING: Air-purifying respirators do not protect workers in oxygen-deficient atmospheres. 
Skin Protection: 
Wear impervious protective clothing, including boots, gloves, lab coat, apron or coveralls, as appropriate, to prevent skin contact. 
Eye Protection: 
Maintain eye wash fountain and quick-drench facilities in work area. Use chemical safety goggles and/or full face shield where dusting or splashing of solutions is possible.

## 9. Physical and Chemical Properties

Appearance: 
Clear, colorless liquid. 
Odor: 
Mild, fruity odor. 
Solubility: 
Slightly soluble in water (ca. 0.7% @ 20C) 
Specific Gravity: 
0.8822 at 20C/20C 
pH: 
No information found. 
% Volatiles by volume @ 21C (70F): 
100 
Boiling Point: 
126C (259F) 
Melting Point: 
-77C (-107F) 
Vapor Density (Air=1): 
4.0 
Vapor Pressure (mm Hg): 
15 @ 25C (77F) 
Evaporation Rate (BuAc=1): 
1

## 10. Stability and Reactivity

Stability: 
Stable under ordinary conditions of use and storage. 
Hazardous Decomposition Products: 
Carbon dioxide and carbon monoxide may form when heated to decomposition. 
Hazardous Polymerization: 
Will not occur. 
Incompatibilities: 
Dangerous when exposed to heat or flame; can react with oxidizing materials, strong alkalis, acids, nitrates and potassium-tert-butoxide. 
Conditions to Avoid: 
Heat, flames, ignition sources and incompatibles.

## 11. Toxicological Information

Toxicological Data: 
Oral rat LD50: 10.8 g/kg;
inhalation rat LC50: 390 ppm/4H
Skin rabbit LD50: >17,600 mg/kg;
Irritant, skin rabbit (Std. Draize): 500 mg/24H, moderate. Irritant, eye rabbit: 100 mg moderate. Investigated as a reproductive effector. 
Reproductive Toxicity: 
Has shown teratogenic effects in laboratory animals.

-\Cancer Lists\------------------------------------------------------

---NTP Carcinogen---

Ingredient                      Known    Anticipated    IARC Category

-----------------------------   -----    -----------    -------------

n-Butyl Acetate (123-86-4)        No          No            None

## 12. Ecological Information

Environmental Fate: 
When released into the soil, this material is expected to readily biodegrade. When released into the soil, this material may leach into groundwater. When released into the soil, this material is expected to have a half-life of less than 1 day. When released into water, this material is expected to readily biodegrade. When released into the water, this material is expected to have a half-life between 1 and 10 days. This material has an estimated bioconcentration factor (BCF) of less than 100. When released into the air, this material may be moderately degraded by reaction with photochemically produced hydroxyl radicals. 
Environmental Toxicity: 
96 Hr LC50 fathead minnow: 18 mg/L (flow-through);
96 Hr LC50 bluegill: 100 mg/L (Static);
96 Hr EC50 freshwater algae (Scenedesmus subspicatus): 320 mg/L;
48 Hr EC50 water flea: 44 mg/L.

## 13. Disposal Considerations

Whatever cannot be saved for recovery or recycling should be handled as hazardous waste and sent to a RCRA approved waste facility. Processing, use or contamination of this product may change the waste management options. State and local disposal regulations may differ from federal disposal regulations. Dispose of container and unused contents in accordance with federal, state and local requirements.

## 14. Transport Information

Domestic (Land, D.O.T.) 
----------------------- 
Proper Shipping Name: BUTYL ACETATES 
Hazard Class: 3 
UN/NA: UN1123 
Packing Group: III 
Information reported for product/size: 400LB 

International (Water, I.M.O.) 
----------------------------- 
Proper Shipping Name: BUTYL ACETATES 
Hazard Class: 3 
UN/NA: UN1123 
Packing Group: III 
Information reported for product/size: 400LB

## 15. Regulatory Information

-\Chemical Inventory Status - Part 1\---------------------------------

Ingredient                                TSCA  EC   Japan  Australia

----------------------------------------  ----  ---  -----  ---------

n-Butyl Acetate (123-86-4)                Yes   Yes   Yes      Yes

-\Chemical Inventory Status - Part 2\---------------------------------

--Canada--

Ingredient                                 Korea  DSL   NDSL  Phil.

-----------------------------------------  -----  ---   ----  -----

n-Butyl Acetate (123-86-4)                  Yes   Yes   No     Yes

-\Federal, State & International Regulations - Part 1\----------------

-SARA 302-    ------SARA 313------

Ingredient                           RQ    TPQ     List  Chemical Catg.

----------------------------------  ---   -----   ----  --------------

n-Butyl Acetate (123-86-4)           No    No      No         No

-\Federal, State & International Regulations - Part 2\----------------

-RCRA-    -TSCA-

Ingredient                                 CERCLA     261.33     8(d)

-----------------------------------------  ------     ------    ------

n-Butyl Acetate (123-86-4)                 5000       No         No

Chemical Weapons Convention:  No     TSCA 12(b):  No     CDTA:  No

SARA 311/312:  Acute: Yes      Chronic: Yes  Fire: Yes Pressure: No

Reactivity: No          (Pure / Liquid)

Australian Hazchem Code: 3[Y]E 
Poison Schedule: None allocated. 
WHMIS: 
This MSDS has been prepared according to the hazard criteria of the Controlled Products Regulations (CPR) and the MSDS contains all of the information required by the CPR.

## 16. Other Information

NFPA Ratings: Health: 2 Flammability: 3 Reactivity: 0 
Label Hazard Warning: 
WARNING! FLAMMABLE LIQUID AND VAPOR. HARMFUL IF SWALLOWED OR INHALED. CAUSES SEVERE IRRITATION TO EYES. CAUSES IRRITATION TO SKIN AND RESPIRATORY TRACT. AFFECTS CENTRAL NERVOUS SYSTEM. 
Label Precautions: 
Keep away from heat, sparks and flame.
Keep container closed.
Use only with adequate ventilation.
Avoid contact with eyes, skin and clothing.
Wash thoroughly after handling.
Avoid breathing vapor. 
Label First Aid: 
In case of contact, immediately flush eyes or skin with plenty of water for at least 15 minutes. Remove contaminated clothing and shoes. Wash clothing before reuse. If inhaled, remove to fresh air. If not breathing, give artificial respiration. If breathing is difficult, give oxygen. If swallowed, induce vomiting immediately as directed by medical personnel. Never give anything by mouth to an unconscious person. In all cases, get medical attention. 
Product Use: 
Laboratory Reagent. 
Revision Information: 
No Changes. 
Disclaimer

*********************************************************************** 
Mallinckrodt Baker, Inc. provides the information contained herein in good faith but makes no representation as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose. MALLINCKRODT BAKER, INC. MAKES NO REPRESENTATIONS OR WARRANTIES, EITHER EXPRESS OR IMPLIED, INCLUDING WITHOUT LIMITATION ANY WARRANTIES OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE WITH RESPECT TO THE INFORMATION SET FORTH HEREIN OR THE PRODUCT TO WHICH THE INFORMATION REFERS. ACCORDINGLY, MALLINCKRODT BAKER, INC. WILL NOT BE RESPONSIBLE FOR DAMAGES RESULTING FROM USE OF OR RELIANCE UPON THIS INFORMATION. 
************************************************************************ 
Prepared by: Environmental Health & Safety
Phone Number: (314) 654-1600 (U.S.A.)